Embedded Systems Lab Setup & Sensor Interfacing

TASK – 2  |  Submission Report

| Field | Details | Info |
| --- | --- | --- |
| Name | [Your Full Name] | Task-2 |
| Internship Role | Embedded System Developer (Trainee) | Maincrafts |
| Sensor Used | DHT11 – Temperature & Humidity Sensor | Arduino UNO |
| Environment | Wokwi Online Simulator / Arduino IDE | C / C++ |

# 1. Objective

The goal of Task-2 is to set up an embedded development environment and gain practical experience in sensor interfacing — the foundation of all IoT and hardware products.

By completing this task, the following outcomes are achieved:

- Set up Arduino IDE (offline) and/or Wokwi online simulator

- Understand how microcontrollers communicate with sensors

- Write a first working embedded C/C++ program

- Build confidence with real-time hardware logic and threshold-based automation

# 2. Working Environment

Environment chosen: Wokwi Simulator + Arduino IDE (offline)

| Environment | Description | Best For |
| --- | --- | --- |
| Real Hardware | Arduino UNO + Sensor + Breadboard | Students with hardware |
| Tinkercad | Browser-based drag-drop circuit + coding | Beginners without hardware |
| Wokwi | Advanced online simulator for ESP32/Arduino | Intermediate learners |

# 3. Sensor Used – DHT11

## 3.1 Overview

The DHT11 is a low-cost digital sensor capable of measuring both temperature (0–50°C) and relative humidity (20–90% RH). It uses a single-wire serial interface to communicate with the Arduino UNO.

## 3.2 Technical Specifications

| Parameter | Value |
| --- | --- |
| Supply Voltage | 3.3V – 5V DC |
| Temperature Range | 0°C to 50°C (±2°C accuracy) |
| Humidity Range | 20% to 90% RH (±5% accuracy) |
| Communication Protocol | Single-wire digital (1-Wire) |
| Sampling Rate | Once every 1 second |
| Pins | VCC, DATA, GND (3 or 4 pin) |

## 3.3 How the DHT11 Sensor Works

The DHT11 sensor contains a capacitive humidity element and a thermistor internally. When the Arduino sends a start signal on the DATA pin, the DHT11 responds with a 40-bit data packet containing humidity (16 bits), temperature (16 bits), and a checksum (8 bits). The Arduino decodes this data using the DHT library and converts the raw values to human-readable temperature (°C) and humidity (% RH).

# 4. Circuit Connections

## 4.1 Components Required

- Arduino UNO (×1)

- DHT11 Temperature & Humidity Sensor (×1)

- Red LED – High Temperature Alert (×1)

- Yellow LED – Normal Status Indicator (×1)

- Piezo Buzzer (×1)

- 220Ω Resistors for LEDs (×2)

- 10kΩ Pull-up Resistor for DHT11 DATA pin (×1)

- Breadboard and Jumper Wires

## 4.2 Pin Connection Table

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| DHT11 | VCC | 5V | Power supply |
| DHT11 | DATA | Digital Pin 2 | 10kΩ pull-up to 5V |
| DHT11 | GND | GND | Ground |
| Red LED | Anode (+) | Digital Pin 8 | 220Ω resistor in series |
| Red LED | Cathode (–) | GND | Ground |
| Yellow LED | Anode (+) | Digital Pin 9 | 220Ω resistor in series |
| Yellow LED | Cathode (–) | GND | Ground |
| Piezo Buzzer | Positive (+) | Digital Pin 10 | Alert buzzer |
| Piezo Buzzer | Negative (–) | GND | Ground |

## 4.3 Circuit Wiring Notes

- Connect a 10kΩ pull-up resistor between the DHT11 DATA pin and 5V — this is required for reliable single-wire communication.

- Always connect a 220Ω resistor in series with each LED to limit current and prevent burning the LED.

- The piezo buzzer can be connected directly to the digital pin; no current-limiting resistor is needed.

- Use the Arduino's 5V and GND rails (power rails on breadboard) for distribution.

# 5. Source Code

## 5.1 Library Used

DHT Sensor Library by Adafruit — Install via Arduino IDE: Sketch → Include Library → Manage Libraries → Search "DHT sensor library"

## 5.2 Complete Arduino Code (.ino)

Smart_Temperature_Monitor.ino

// ============================================================

//  Smart Temperature & Humidity Monitor

//  Sensor  : DHT11

//  Platform: Arduino UNO

//  Task    : Maincrafts Technology – Embedded Systems Task 2

// ============================================================

#include <DHT.h>

// ── Pin Definitions ─────────────────────────────────────────

#define DHTPIN       2     // DHT11 DATA pin → Arduino D2

#define DHTTYPE      DHT11 // Sensor type

#define RED_LED      8     // High-temp alert LED

#define YELLOW_LED   9     // Normal-status LED

#define BUZZER       10    // Piezo buzzer

// ── Threshold ────────────────────────────────────────────────

#define TEMP_THRESHOLD  30.0   // Alert if temp > 30°C

#define HUM_THRESHOLD   70.0   // Alert if humidity > 70%

DHT dht(DHTPIN, DHTTYPE);

// ── setup() ──────────────────────────────────────────────────

void setup() {

Serial.begin(9600);

dht.begin();

pinMode(RED_LED,    OUTPUT);

pinMode(YELLOW_LED, OUTPUT);

pinMode(BUZZER,     OUTPUT);

// Startup indicator

digitalWrite(YELLOW_LED, HIGH);

delay(1000);

digitalWrite(YELLOW_LED, LOW);

Serial.println("=== Smart Temperature Monitor ===");

Serial.println("Monitoring started...");

}

// ── loop() ───────────────────────────────────────────────────

void loop() {

delay(2000);   // DHT11 sampling interval

float humidity    = dht.readHumidity();

float temperature = dht.readTemperature(); // Celsius

// Check for sensor read failure

if (isnan(humidity) || isnan(temperature)) {

Serial.println("[ERROR] DHT11 read failed!");

return;

}

// ── Print to Serial Monitor ──────────────────────────────

Serial.print("Temperature : ");

Serial.print(temperature);

Serial.println(" °C");

Serial.print("Humidity    : ");

Serial.print(humidity);

Serial.println(" %");

Serial.println("----------------------------");

// ── Threshold Logic ──────────────────────────────────────

if (temperature > TEMP_THRESHOLD || humidity > HUM_THRESHOLD) {

// HIGH ALERT: temperature or humidity exceeded

digitalWrite(RED_LED,    HIGH);

digitalWrite(YELLOW_LED, LOW);

tone(BUZZER, 1000);           // 1 kHz alert tone

Serial.println(">> ALERT: High Temperature / Humidity!");

} else {

// NORMAL: within safe limits

digitalWrite(RED_LED,    LOW);

digitalWrite(YELLOW_LED, HIGH);

noTone(BUZZER);

Serial.println(">> Status: NORMAL");

}

}

// ── End of Program ───────────────────────────────────────────

# 6. Code Explanation

| Code Section | What It Does |
| --- | --- |
| #include <DHT.h> | Loads the Adafruit DHT library for sensor communication |
| #define DHTPIN / DHTTYPE | Assigns sensor data pin (D2) and sensor model (DHT11) |
| TEMP_THRESHOLD | Alert trigger level — 30°C. Easily adjustable without re-wiring |
| setup() | Initializes Serial (9600 baud), sensor, pin modes, and startup blink |
| dht.readTemperature() | Reads temperature in Celsius from DHT11 via 1-Wire protocol |
| isnan() check | Validates sensor data; prevents crash if DHT11 disconnects or fails |
| if (temp > THRESHOLD) | Threshold comparison — triggers Red LED + Buzzer on alert, Yellow LED on normal |
| tone() / noTone() | Generates 1 kHz alert sound on buzzer; noTone() silences it |
| delay(2000) | DHT11 requires ≥1 second between reads; 2s ensures stable data |

# 7. How to Run / Test

## Option A – Wokwi Simulator (Recommended)

- Open wokwi.com and create a new Arduino UNO project

- Add components: DHT11, 2 LEDs (220Ω resistors), Piezo Buzzer, Breadboard

- Connect wires as per the Pin Connection Table in Section 4.2

- Paste the source code from Section 5.2 into the editor

- Click ▶ Start Simulation and open the Serial Monitor to observe readings

- Adjust the DHT11 temperature slider above 30°C — Red LED and Buzzer activate

## Option B – Real Hardware (Arduino UNO)

- Connect the circuit on a breadboard as per Section 4.2

- Open Arduino IDE, install the DHT Sensor Library (Manage Libraries)

- Paste the code, select Board: Arduino UNO, select the correct COM Port

- Click Upload (→). Open Serial Monitor at 9600 baud to view live readings

- Breathe on the DHT11 or apply heat to test the threshold alert

# 8. Expected Output

## 8.1 Serial Monitor Output (Normal Condition)

=== Smart Temperature Monitor ===

Monitoring started...

Temperature : 27.00 °C

Humidity    : 55.00 %

----------------------------

>> Status: NORMAL

Temperature : 28.00 °C

Humidity    : 58.00 %

----------------------------

>> Status: NORMAL

## 8.2 Serial Monitor Output (Alert Condition – Temp > 30°C)

Temperature : 33.00 °C

Humidity    : 72.00 %

----------------------------

>> ALERT: High Temperature / Humidity!

## 8.3 Physical Outputs

| Condition | LED Status | Buzzer |
| --- | --- | --- |
| Temperature ≤ 30°C, Humidity ≤ 70% | Yellow LED ON | Silent |
| Temperature > 30°C OR Humidity > 70% | Red LED ON | Beeping (1 kHz tone) |
| Sensor disconnected / error | Both OFF | Silent (error printed) |

# 9. Learnings & Observations

- Understood the single-wire (1-Wire) communication protocol used by DHT11

- Learned the importance of a 10kΩ pull-up resistor on the DATA line for signal stability

- Practised threshold-based logic to trigger real-time hardware actions (LED, Buzzer)

- Learned error handling with isnan() to make the program robust against sensor failures

- Gained hands-on experience setting up Wokwi Simulator and Arduino IDE workflow

- Understood the role of delay(2000) for respecting DHT11's minimum sampling interval

# 10. Deliverables Checklist

| # | Deliverable | Status |
| --- | --- | --- |
| 1 | Circuit Diagram (Wokwi / Tinkercad screenshot) | ✔ Screenshot attached |
| 2 | Source Code (.ino file / screenshot) | ✔ Code in Section 5 |
| 3 | Short explanation (sensor + code logic) | ✔ Sections 3, 6 |
| 4 | Demo video 20–30 sec (optional but preferred) | ⬜ To be recorded |

# 11. Resources Used

Arduino IDE: https://www.arduino.cc/en/software

Wokwi Simulator: https://wokwi.com/

Tinkercad: https://www.tinkercad.com/circuits

DHT11 Tutorial Video: https://www.youtube.com/watch?v=E_4qjqCfTGQ

Arduino Setup Video: https://www.youtube.com/watch?v=1XQ0MUYdwxA

Declaration

I hereby declare that this Task-2 report is my own original work completed as part of the Maincrafts Technology Embedded Systems Internship. The circuit, code, and explanations are prepared individually.

Name: ___________________________

Date: ___________________________

Signature: ______________________